# Security in Computing Practical 1

Configure Routers for Syslog, NTP, OSPF MD5 and SSH

Practical 1: Configure Routers for Syslog, NTP and SSH operation

Topology:

# Router R1 Configuration

enable
conf t
hostname R1
enable secret enpa55
line vty 0 4
 password vtypa55
 login
exit
interface g0/0
 ip address 192.168.1.1 255.255.255.0
 no shutdown
exit
interface s0/1/0
 ip address 10.1.1.1 255.255.255.252
 clock rate 64000
 no shutdown
exit
router ospf 1
 network 192.168.1.0 0.0.0.255 area 0
 network 10.1.1.0 0.0.0.3 area 0
 area 0 authentication message-digest
exit
interface s0/1/0
 ip ospf message-digest-key 1 md5 MD5pa55
exit
ntp server 192.168.1.5
ntp update-calendar
ntp authenticate
ntp trusted-key 1
ntp authentication-key 1 md5 NTPpa55
service timestamps log datetime msec
logging host 192.168.1.6
end
wr

# Router R2 Configuration

enable
conf t
hostname R2
enable secret enpa55
line vty 0 4
 password vtypa55
 login
exit
interface s0/1/0
 ip address 10.1.1.2 255.255.255.252
 no shutdown
exit
interface s0/1/1
 ip address 10.2.2.2 255.255.255.252
 no shutdown
exit
router ospf 1
 network 10.1.1.0 0.0.0.3 area 0
 network 10.2.2.0 0.0.0.3 area 0
 area 0 authentication message-digest
exit
interface s0/1/0
 ip ospf message-digest-key 1 md5 MD5pa55
exit
interface s0/1/1
 ip ospf message-digest-key 1 md5 MD5pa55
exit
ntp server 192.168.1.5
ntp update-calendar
ntp authenticate
ntp trusted-key 1
ntp authentication-key 1 md5 NTPpa55
service timestamps log datetime msec
logging host 192.168.1.6
end
wr

# Router R3 Configuration

enable
conf t
hostname R3
enable secret enpa55
ip domain-name ccnasecurity.com
username SSHadmin privilege 15 secret sshpa55
line vty 0 4
 login local
 transport input ssh
exit
interface g0/0
 ip address 192.168.3.1 255.255.255.0
 no shutdown
exit
interface s0/1/0
 ip address 10.2.2.1 255.255.255.252
 no shutdown
exit
router ospf 1
 network 192.168.3.0 0.0.0.255 area 0
 network 10.2.2.0 0.0.0.3 area 0
 area 0 authentication message-digest
exit
interface s0/1/0
 ip ospf message-digest-key 1 md5 MD5pa55
exit
ntp server 192.168.1.5
ntp update-calendar
ntp authenticate
ntp trusted-key 1
ntp authentication-key 1 md5 NTPpa55
service timestamps log datetime msec
logging host 192.168.1.6
crypto key zeroize rsa
crypto key generate rsa
1024
ip ssh time-out 90
ip ssh authentication-retries 2
ip ssh version 2
end
wr

# Verification Steps

1. Check interfaces:
show ip interface brief

2. Check OSPF neighbors:
show ip ospf neighbor

3. Test connectivity:
ping 192.168.3.1 (from R1)

4. Test SSH:
ssh -l SSHadmin 192.168.3.1

5. Check logs:
show logging

6. Check NTP:
show ntp status

Practical 1: Syslog, NTP, and SSH

- On R1, R2, and R3:

  - show ntp status: To verify if the clock is synchronized with the NTP server (192.168.1.5).

  - show ntp associations: To see the status of the connection to the NTP server.

  - show logging: To confirm that system messages are being time-stamped and sent to the Syslog host (192.168.1.6).

- On R1:

  - ping 192.168.3.1: To test end-to-end connectivity to R3 via OSPF.

- From a PC or R1:

  - ssh -l SSHadmin 192.168.3.1: To verify SSH access to R3 using the configured username.

Practical 1: Syslog, NTP, and SSH

On Routers (R1, R2, R3)

- Verify OSPF MD5: Run show ip ospf interface. Look for "Message-digest Authentication Enabled" and "Youngest key ID is 1".

- Verify NTP Client: Run show ntp status to check synchronization with the server (192.168.1.5).

- Verify Hardware Clock: Run show clock to ensure the time has updated via NTP.

- Verify Logging: Run show logging to confirm that the Syslog host is configured.

- Verify SSH (on R3): Run show ip ssh to confirm Version 2, Timeout 90, and Retries 2.

On PCs (PC-A, PC-B, PC-C)

- Connectivity: From PC-A, run ping 192.168.3.5 (PC-C).

- SSH Access: From PC-C, run ssh -l SSHadmin 192.168.3.1 and enter password sshpa55.

- Telnet Block: From PC-C, run telnet 192.168.3.1 (should fail).