# Security in Computing Practical 2

Configure AAA Authentication on Cisco Router (R1)

Practical 2: Configure AAA Authentication on Cisco routers

Topology:

# Router R1 – Full Configuration

enable
conf t

hostname R1

enable secret enpa55

line vty 0 4
 password vtypa55
 login
exit

interface g0/0
 ip address 192.168.1.1 255.255.255.0
 no shutdown
exit

router ospf 1
 network 192.168.1.0 0.0.0.255 area 0
 area 0 authentication message-digest
exit

interface g0/0
 ip ospf message-digest-key 1 md5 pa55
exit

username admin secret adminpa55
aaa new-model
aaa authentication login default local

line console 0
 login authentication default
exit

ip domain-name ccnasecurity.com
crypto key generate rsa
1024

aaa authentication login SSH-LOGIN local

line vty 0 4
 login authentication SSH-LOGIN
 transport input ssh
exit

end
wr

# Verification Steps

1. show ip interface brief
2. show ip ospf interface
3. ping between PCs
4. Login using:
Username: admin
Password: adminpa55
5. ssh -l admin 192.168.1.1
6. show ip ssh

Practical 2: AAA Authentication

- On R1:

  - show running-config | section aaa: To verify that aaa new-model and authentication lists are active.

  - show ip ssh: To check the SSH version and authentication settings.

- Testing Login:

  - Console/VTY Login: Attempt to log in; the router should prompt for the username admin and password adminpa55.

  - ssh -l admin 192.168.1.1: To test the AAA-controlled SSH access.

Practical 2: Local AAA Authentication

On Router R1

- Console Verification: Exit the session. When you press Enter, it must prompt for Username: admin and Password: adminpa55.

On PCs (PC0, PC1)

- VTY Verification: Run ssh -l Admin 192.168.1.1. It should authenticate using the local database.

# Viva Questions

AAA = Authentication, Authorization, Accounting
Authentication = verifies user identity
SSH is secure, Telnet is not
Local AAA uses router database