# Security in Computing Practical 3

Configuring Extended ACLs

Practical 3: Configuring Extended ACLs

A]

Topology:

# Part A – Router R1 Configuration (Extended ACL)

enable
conf t

hostname R1

enable secret enpa55

line vty 0 4
 password vtypa55
 login
exit

! Interface configuration
interface g0/0
 ip address 172.22.34.65 255.255.255.224
 no shutdown
exit

interface g0/1
 ip address 172.22.34.97 255.255.255.240
 no shutdown
exit

interface g0/2
 ip address 172.22.34.1 255.255.255.192
 no shutdown
exit

! Extended Numbered ACL (PC1 → FTP only)
access-list 100 permit tcp 172.22.34.64 0.0.0.31 host 172.22.34.62 eq ftp
access-list 100 permit icmp 172.22.34.64 0.0.0.31 host 172.22.34.62

interface g0/0
 ip access-group 100 in
exit

! Extended Named ACL (PC2 → HTTP only)
ip access-list extended HTTP_ONLY
 permit tcp 172.22.34.96 0.0.0.15 host 172.22.34.62 eq www
 permit icmp 172.22.34.96 0.0.0.15 host 172.22.34.62
exit

interface g0/1
 ip access-group HTTP_ONLY in
exit

end
wr

# Part B – Router RTA Configuration

enable
conf t

hostname RTA

enable secret enpa55

line console 0
 password tyit
 login
exit

! Interface configuration
interface g0/0
 ip address 10.101.117.49 255.255.255.248
 no shutdown
exit

interface g0/1
 ip address 10.101.117.33 255.255.255.240
 no shutdown
exit

interface g0/2
 ip address 10.101.117.1 255.255.255.224
 no shutdown
exit

! SSH configuration
ip domain-name ccnasecurity.com
username admin secret adminpa55
crypto key generate rsa
1024

line vty 0 4
 login local
 transport input ssh
exit

! ACL for SSH only + ICMP
access-list 199 permit tcp 10.101.117.32 0.0.0.15 10.101.117.0 0.0.0.31 eq 22
access-list 199 permit icmp any any

interface g0/2
 ip access-group 199 out
exit

end
wr

# Switch Configuration (SWA, SWB, SWC)

! SWA
conf t
interface vlan 1
 ip address 10.101.117.50 255.255.255.248
 no shutdown
exit
ip default-gateway 10.101.117.49

! SWB
interface vlan 1
 ip address 10.101.117.34 255.255.255.240
 no shutdown
exit
ip default-gateway 10.101.117.33

! SWC
interface vlan 1
 ip address 10.101.117.2 255.255.255.224
 no shutdown
exit
ip default-gateway 10.101.117.1

# Verification Steps

1. show ip interface brief
2. show access-lists

3. Test from PC1:
- ping server (should work)
- ftp server (should work)
- ping PC2 (should fail)

4. Test from PC2:
- ping server (should work)
- http server (should work)
- ftp server (should fail)

5. SSH Tests (Part B):
ssh -l admin <IP>

6. Ping allowed but SSH restricted (as per ACL)

Practical 3: Extended ACLs

- On R1:

  - show access-lists: To see the hit count for the numbered ACL (100) and named ACL (HTTP_ONLY).

  - show ip interface g0/0: To verify that access-group 100 is applied "inbound".

- On RTA:

  - show access-lists 199: To verify the SSH and ICMP permit rules.

- Test Connectivity:

  - From PC1: ping the server (success) and ftp the server (success).

  - From PC2: ping the server (success) and attempt http access (success), but ftp should fail.

Practical 3: Extended ACLs

On PC1

- FTP Access: Run ftp 172.22.34.62 (should be successful).

- ICMP Access: Run ping 172.22.34.62 (should be successful).

- Denied Access: Run ping 172.22.34.98 (should show "destination host unreachable").

On PC2

- Web Access: Open Web Browser and go to http://172.22.34.62 (successful).

- FTP Block: Run ftp 172.22.34.62 (should be unsuccessful).

# Viva Questions

1. What is ACL?
Access Control List is used to filter network traffic.

2. Difference between Standard and Extended ACL?
Standard → based on source IP
Extended → based on source, destination, protocol, port

3. Why use ACL?
To control and secure network traffic.

4. What is Named ACL?
ACL with a name instead of number.

5. What is port 22?
SSH protocol port.