# Security in Computing Practical 6

Configuring a Zone-Based Policy Firewall (ZPF)

Practical 6:- Configuring a Zone-Based Policy Firewall (ZPF)

Topology:

# Router R1 Configuration

enable
conf t
hostname R1
enable secret enpa55

line console 0
 password conpa55
 login
exit

line vty 0 4
 password vtypa55
 login
exit

ip domain-name ccnasecurity.com
username admin secret adminpa55

line vty 0 4
 login local
 transport input ssh
exit

crypto key generate rsa
1024

interface g0/0
 ip address 192.168.1.1 255.255.255.0
 no shutdown
exit

interface s0/1/0
 ip address 10.1.1.1 255.255.255.252
 clock rate 64000
 no shutdown
exit

ip route 192.168.3.0 255.255.255.0 10.1.1.2

end
wr

# Router R2 Configuration

enable
conf t
hostname R2
enable secret enpa55

line console 0
 password conpa55
 login
exit

line vty 0 4
 password vtypa55
 login
exit

ip domain-name ccnasecurity.com
username admin secret adminpa55

line vty 0 4
 login local
 transport input ssh
exit

crypto key generate rsa
1024

interface s0/1/0
 ip address 10.1.1.2 255.255.255.252
 no shutdown
exit

interface s0/1/1
 ip address 10.2.2.2 255.255.255.252
 clock rate 64000
 no shutdown
exit

! Static routes (IMPORTANT - your missing part added)
ip route 192.168.1.0 255.255.255.0 10.1.1.1
ip route 192.168.3.0 255.255.255.0 10.2.2.1

end
wr

# Router R3 Configuration (ZPF Firewall)

enable
conf t
hostname R3
enable secret enpa55

line console 0
 password conpa55
 login
exit

line vty 0 4
 password vtypa55
 login
exit

ip domain-name ccnasecurity.com
username admin secret adminpa55

line vty 0 4
 login local
 transport input ssh
exit

crypto key generate rsa
1024

interface g0/0
 ip address 192.168.3.1 255.255.255.0
 no shutdown
exit

interface s0/1/0
 ip address 10.2.2.1 255.255.255.252
 no shutdown
exit

! ZONES
zone security IN-ZONE
zone security OUT-ZONE

! ACL for internal traffic
access-list 101 permit ip 192.168.3.0 0.0.0.255 any

! CLASS MAP
class-map type inspect match-all IN-NET-CLASS-MAP
 match access-group 101

! POLICY MAP
policy-map type inspect IN-2-OUT-PMAP
 class type inspect IN-NET-CLASS-MAP
  inspect

! ZONE PAIR
zone-pair security IN-2-OUT-ZPAIR source IN-ZONE destination OUT-ZONE
 service-policy type inspect IN-2-OUT-PMAP

! ASSIGN INTERFACES
interface g0/0
 zone-member security IN-ZONE
exit

interface s0/1/0
 zone-member security OUT-ZONE
exit

ip route 192.168.1.0 255.255.255.0 10.2.2.2

end
wr

# Verification Steps

1. show ip interface brief
2. show ip route
3. show policy-map type inspect zone-pair sessions

4. Test:
PC-C → PC-A (Ping, SSH, HTTP) → Allowed
PC-A → PC-C → Blocked

5. SSH Test:
ssh -l admin 10.2.2.2

6. Web Test:
http://192.168.1.3

Practical 6: Zone-Based Policy Firewall (ZPF)

- On R3 (Firewall Router):

  - show policy-map type inspect zone-pair sessions: To see active "inspect" sessions created by internal traffic.

  - show zone security: To verify the IN-ZONE and OUT-ZONE memberships.

  - show class-map type inspect: To see the traffic matching criteria (ACL 101).

- Test Connectivity:

  - From PC-C (Inside): Ping/SSH to PC-A (Outside) should work.

  - From PC-A (Outside): Ping to PC-C (Inside) should be blocked by default.

Practical 6: Zone-Based Policy Firewall (ZPF)

On Router R3

- Inspect Sessions: While PC-C is pinging or using SSH to R2, run show policy-map type inspect zone-pair sessions. It should show an active session.

On PC-A (Outside Zone)

- Inbound Block: Run ping 192.168.3.3 (should time out because traffic from OUT-ZONE to IN-ZONE is not permitted).

# Viva Questions

1. What is ZPF?
Zone-Based Policy Firewall is used to control traffic between zones.

2. What is Zone?
Logical grouping of interfaces.

3. What is Class-map?
Defines traffic.

4. What is Policy-map?
Defines action (inspect, drop, pass).

5. Why ZPF?
More secure than ACL.