# 🔹 PART 1 – OSPF MD5

## 📍 Path:

R1 → CLI

enable

configure terminal

router ospf 1

area 0 authentication message-digest

interface s0/0/0

ip ospf message-digest-key 1 md5 MD5pa55

end

## 📍 Path:

R2 → CLI

enable

configure terminal

router ospf 1

area 0 authentication message-digest

interface s0/0/0

ip ospf message-digest-key 1 md5 MD5pa55

interface s0/0/1

ip ospf message-digest-key 1 md5 MD5pa55

end

## 📍 Path:

R3 → CLI

enable

configure terminal

router ospf 1

area 0 authentication message-digest

interface s0/0/1

ip ospf message-digest-key 1 md5 MD5pa55

end

## 🔹 VERIFY

📍 Path:

R1 / R2 / R3 → CLI

show ip ospf interface

# 🔹 PART 2 – NTP

## 📍 Path:

R1 → CLI

configure terminal

ntp server 192.168.1.5

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 NTPpa55

ntp update-calendar

end

## 📍 Path:

R2 → CLI

configure terminal

ntp server 192.168.1.5

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 NTPpa55

ntp update-calendar

end

## 📍 Path:

R3 → CLI

configure terminal

ntp server 192.168.1.5

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 NTPpa55

ntp update-calendar

end

## 🔹 VERIFY

show ntp status

show clock

# 🔹 PART 3 – SYSLOG

## 📍 Path:

R1 → CLI

configure terminal

service timestamps log datetime

logging 192.168.1.5

end

## 📍 Path:

R2 → CLI

configure terminal

service timestamps log datetime

logging 192.168.1.5

end

## 📍 Path:

R3 → CLI

configure terminal

service timestamps log datetime

logging 192.168.1.5

end

## 🔹 VERIFY

show logging

# 🔹 PART 4 – SSH (R3)

## 📍 Path:

R3 → CLI

enable

configure terminal

ip domain-name ccnasecurity.com

username SSHadmin privilege 15 secret ciscosshpa55

line vty 0 4

login local

transport input ssh

exit

crypto key zeroize rsa

crypto key generate rsa

1024

ip ssh time-out 90

ip ssh authentication-retries 2

ip ssh version 2

end

## 🔹 VERIFY SSH

show ip ssh

# 🔹 TEST (IMPORTANT)

## 📍 Path:

PC-C → Desktop → Command Prompt

telnet 192.168.3.1

❌ Should fail

ssh -l SSHadmin 192.168.3.1

✔️ Enter password:

ciscosshpa55

## 📍 Path:

R2 → CLI

ssh -v 2 -l SSHadmin 10.2.2.1

# 🎯 FINAL

👉 All commands = manual based
👉 Paste exactly at given path
👉 This gives 100% ✔️

If anything fails, tell me which part 👍