Great 👍 — I will follow ONLY your manual and explain:

👉 Simple Hienglish concept
👉 Same manual commands
👉 Where to type
👉 How to check output (VERY IMPORTANT)

# 🌐 🔥 WHAT IS THIS TOPIC? (Extended ACL)

👉 ACL = Access Control List

### Simple meaning:

👉 Network me kaun allow hai aur kaun block hai decide karta hai

# 🧠 🔹 SCENARIO (VERY SIMPLE)

- PC1 → only FTP + Ping allowed

- PC2 → only Web (HTTP) + Ping allowed

- PC1 & PC2 → ek dusre ko access nahi

# 🔹 PART 1 – NUMBERED ACL (PC1 → FTP + ICMP)

📍 Path:

R1 → CLI

## 💻 👉 TYPE (Same manual code)

enable

configure terminal

access-list 100 permit tcp 172.22.34.64 0.0.0.31 host 172.22.34.62 eq ftp

access-list 100 permit icmp 172.22.34.64 0.0.0.31 host 172.22.34.62

interface gigabitEthernet 0/0

ip access-group 100 in

end

## 🧠 SIMPLE EXPLANATION

access-list 100 permit tcp ...

👉 PC1 network → Server → only FTP allow

access-list 100 permit icmp ...

👉 Ping allow

👉 Baaki sab ❌ block (default)

# ✅ 🔍 HOW TO CHECK OUTPUT (PART 1)

## ✔️ 1. Ping Test

📍 Path:

PC1 → Desktop → Command Prompt

👉 Type:

ping 172.22.34.62

### ✔️ Correct Output:

Reply from ...

## ✔️ 2. FTP Test

👉 Type:

ftp 172.22.34.62

👉 Enter:

Username: cisco

Password: cisco

### ✔️ Correct Output:

ftp>

👉 Means FTP allowed ✅

## ✔️ 3. PC1 → PC2 (SHOULD FAIL)

ping 172.22.34.98

### ❌ Expected Output:

Destination host unreachable

👉 Means ACL working ✅

# 🔹 PART 2 – NAMED ACL (PC2 → HTTP + ICMP)

📍 Path:

R1 → CLI

## 💻 👉 TYPE (Same manual code)

configure terminal

ip access-list extended HTTP_ONLY

permit tcp 172.22.34.96 0.0.0.15 host 172.22.34.62 eq www

permit icmp 172.22.34.96 0.0.0.15 host 172.22.34.62

interface gigabitEthernet 0/1

ip access-group HTTP_ONLY in

end

## 🧠 SIMPLE EXPLANATION

permit tcp ... eq www

👉 PC2 → Server → only web (port 80)

permit icmp ...

👉 Ping allow

# ✅ 🔍 HOW TO CHECK OUTPUT (PART 2)

## ✔️ 1. Ping Test

📍 Path:

PC2 → Desktop → Command Prompt

ping 172.22.34.62

👉 ✔️ Should work

## ✔️ 2. FTP Test (SHOULD FAIL)

ftp 172.22.34.62

👉 ❌ Should fail

## ✔️ 3. WEB TEST

📍 Path:

PC2 → Desktop → Web Browser

👉 Enter:

172.22.34.62

👉 ✔️ Page should open

# 🔥 FINAL UNDERSTANDING

| Device | Allowed | Blocked |
| --- | --- | --- |
| PC1 | FTP + Ping | HTTP, others |
| PC2 | HTTP + Ping | FTP, others |

# 🎯 HOW TO KNOW EVERYTHING IS CORRECT

✔️ PC1 → FTP works
✔️ PC1 → PC2 ping fails
✔️ PC2 → Web works
✔️ PC2 → FTP fails

👉 Then = ✅ Output correct (as per manual)

If you want, I can give you short exam answer (write in paper directly) 👍