# 🌐 🔥 WHAT IS THIS SCENARIO?

👉 Rule:

- ✔️ PCB → Telnet allowed

- ❌ PCA → Telnet NOT allowed

- ✔️ ICMP (ping) → allowed for all

- ❌ All other traffic → blocked

# 🔹 PART 1 – EXTENDED NUMBERED ACL

📍 Path:

RTA → CLI

## 💻 👉 STEP 1a (First ACL Statement)

👉 Manual logic se final command:

access-list 199 permit tcp 10.101.117.32 0.0.0.15 10.101.117.0 0.0.0.31 eq telnet

## 🧠 SIMPLE EXPLANATION

- tcp → Telnet uses TCP

- 10.101.117.32 0.0.0.15 → PCB network

- 10.101.117.0 0.0.0.31 → destination network

- eq telnet → only telnet allow

## 💻 👉 STEP 1b (Second Statement)

access-list 199 permit icmp any any

👉 Means:
✔️ Ping allowed for everyone

# 🔹 STEP 2 – APPLY ACL

👉 Manual says: apply on G0/2 outbound

interface gigabitEthernet 0/2

ip access-group 199 out

# 🔥 FULL CODE (FINAL)

enable

configure terminal

access-list 199 permit tcp 10.101.117.32 0.0.0.15 10.101.117.0 0.0.0.31 eq telnet

access-list 199 permit icmp any any

interface gigabitEthernet 0/2

ip access-group 199 out

end

# ✅ 🔍 HOW TO CHECK OUTPUT (VERY IMPORTANT)

## ✔️ 1. Ping Test (ALL SHOULD WORK)

📍 Path:

PCA / PCB → Desktop → Command Prompt

ping 10.101.117.2

👉 ✔️ Should work

## ✔️ 2. TELNET TEST (MAIN CHECK)

### 👉 From PCB (ALLOWED)

📍 Path:

PCB → Command Prompt

telnet 10.101.117.2

👉 Enter:

Password: cisco

### ✔️ Expected:

User Access Verification

👉 Means ✅ Allowed

### 👉 From PCA (BLOCKED)

📍 Path:

PCA → Command Prompt

telnet 10.101.117.2

### ❌ Expected:

Connection failed / refused

👉 Means ✅ ACL working

## ✔️ 3. IMPORTANT TEST (BYPASS CASE)

👉 Manual step:

- PCA → telnet SWB (allowed)

- Then SWB → telnet SWC (allowed)

👉 ✔️ This will WORK

# 🔥 PART 2 – ANSWERS (EXAM STYLE)

## ❓ Q1. PCA bypass kaise hua?

👉 Answer:
👉 PCA directly blocked tha
👉 But PCA → SWB → SWC gaya

👉 Router ne is indirect traffic ko block nahi kiya

## ❓ Q2. Kaise prevent kare?

👉 Answer:
👉 ACL ko aur strict banana hoga

👉 Example:

- SWB par bhi ACL lagana

- Ya destination specific deny rule add karna

# 🎯 FINAL CHECK (IMPORTANT)

| Test | Expected |
| --- | --- |
| PCB → Telnet | ✔️ Allowed |
| PCA → Telnet | ❌ Blocked |
| All Ping | ✔️ Allowed |

# 🔥 FINAL LINE (EXAM)

👉 “Extended ACL controls traffic based on source, destination and protocol.”

If you want, I can give you short 10-mark answer ready for exam writing 👍