# 🌐 WHAT IS THIS SCENARIO (Simple)

Is lab me Named Extended ACL use karna hai.

Rules:

- ❌ PC1 → HTTP / HTTPS block to Server1 & Server2

- ❌ PC2 → FTP block to Server1 & Server2

- ❌ PC3 → Ping block to Server1 & Server2

- ✔️ Baaki sab traffic allowed

# 🔹 PART 1 – CONFIGURE NAMED ACL

📍 Path

RT1 → CLI

# STEP 1 – Start Named ACL

👉 Question: command to begin named ACL

enable

configure terminal

ip access-list extended ACL

# STEP 1b – PC1 HTTP Block (Server1)

PC1 IP = 172.31.1.101
Server1 IP = 64.101.255.254

deny tcp host 172.31.1.101 host 64.101.255.254 eq 80

# STEP 1c – PC1 HTTPS Block (Server1)

deny tcp host 172.31.1.101 host 64.101.255.254 eq 443

# STEP 1d – PC1 HTTP Block (Server2)

Server2 IP = 64.103.255.254

deny tcp host 172.31.1.101 host 64.103.255.254 eq 80

# STEP 1e – PC1 HTTPS Block (Server2)

deny tcp host 172.31.1.101 host 64.103.255.254 eq 443

# STEP 2 – PC2 FTP Block

PC2 IP = 172.31.1.102

### Server1 FTP Block

deny tcp host 172.31.1.102 host 64.101.255.254 eq 21

### Server2 FTP Block

deny tcp host 172.31.1.102 host 64.103.255.254 eq 21

# STEP 3 – PC3 Ping Block

PC3 IP = 172.31.1.103

### Server1

deny icmp host 172.31.1.103 host 64.101.255.254

### Server2

deny icmp host 172.31.1.103 host 64.103.255.254

# STEP 4 – Permit All Other Traffic

👉 Question: command?

permit ip any any

# 🔥 FINAL FULL CODE (IMPORTANT)

Type in RT1 CLI

enable

configure terminal

ip access-list extended ACL

deny tcp host 172.31.1.101 host 64.101.255.254 eq 80

deny tcp host 172.31.1.101 host 64.101.255.254 eq 443

deny tcp host 172.31.1.101 host 64.103.255.254 eq 80

deny tcp host 172.31.1.101 host 64.103.255.254 eq 443

deny tcp host 172.31.1.102 host 64.101.255.254 eq 21

deny tcp host 172.31.1.102 host 64.103.255.254 eq 21

deny icmp host 172.31.1.103 host 64.101.255.254

deny icmp host 172.31.1.103 host 64.103.255.254

permit ip any any

end

# 🔹 PART 2 – APPLY ACL

Traffic LAN se aa raha hai
Isliye ACL lagayenge G0/0 interface inbound

📍 Path

RT1 → CLI

Command:

configure terminal

interface gigabitEthernet 0/0

ip access-group ACL in

end

# 🔍 HOW TO VERIFY OUTPUT (VERY IMPORTANT)

# TEST 1 – PC1 WEB TEST

📍 Path

PC1 → Desktop → Web Browser

Test:

http://64.101.255.254

https://64.101.255.254

Expected:

❌ Should NOT open

# TEST 2 – PC1 FTP TEST

PC1 → Command Prompt

ftp 64.101.255.254

Expected:

✔️ Should connect

# TEST 3 – PC1 PING

ping 64.101.255.254

Expected:

✔️ Should work

# TEST 4 – PC2 FTP TEST

PC2 → Command Prompt

ftp 64.101.255.254

Expected:

❌ Should fail

# TEST 5 – PC3 PING TEST

PC3 → Command Prompt

ping 64.101.255.254

Expected:

❌ Should fail

# 🎯 FINAL RESULT TABLE

| Device | Test | Result |
| --- | --- | --- |
| PC1 | HTTP | Block |
| PC1 | FTP | Allowed |
| PC2 | FTP | Block |
| PC3 | Ping | Block |

# 🔥 VERY IMPORTANT CHECK (Exam Trick)

Type in router:

show access-lists

If ACL working → counters increase

Example output:

ACL

deny tcp host 172.31.1.101 host 64.101.255.254 eq 80 (5 matches)

Means ✔️ Correct

If you want, I can also give you
✔️ Most common mistake students make in this lab (very useful for exam).