# 🌐 🔥 WHAT IS THIS PRACTICAL?

👉 Goal:

- Sirf PC-C (management PC) ko router access mile

- Baaki sab block

- ACL use karke security (attack mitigation) karna

# 🔹 PART 1 – CONNECTIVITY CHECK

📍 Path:

PC-A / PC-C → Desktop → Command Prompt

### ✔️ From PC-A

ping 192.168.3.3

ssh -l SSHadmin 192.168.2.1

Password:

ciscosshpa55

### ✔️ From PC-C

ping 192.168.1.3

ssh -l SSHadmin 192.168.2.1

👉 ✔️ Should work

# 🔹 PART 2 – ACL 10 (ONLY PC-C ACCESS)

📍 Path:

R1 / R2 / R3 → CLI

## 💻 👉 TYPE (on ALL routers)

enable

configure terminal

access-list 10 permit 192.168.3.3

access-list 10 deny any

line vty 0 4

access-class 10 in

end

## 🔍 VERIFY

### ✔️ From PC-C

ssh -l SSHadmin 192.168.2.1

👉 ✔️ Should work

### ❌ From PC-A

ssh -l SSHadmin 192.168.2.1

👉 ❌ Should fail

# 🔹 PART 3 – ACL 120 (R1)

📍 Path:

R1 → CLI

## 💻 👉 TYPE

enable

configure terminal

access-list 120 permit tcp any host 192.168.1.3 eq 53

access-list 120 permit tcp any host 192.168.1.3 eq 25

access-list 120 permit tcp any host 192.168.1.3 eq 21

access-list 120 deny tcp any host 192.168.1.3 eq 443

access-list 120 permit tcp host 192.168.3.3 host 192.168.1.1 eq 22

interface s0/0/0

ip access-group 120 in

end

## 🔍 VERIFY

📍 PC-C → Browser:

https://192.168.1.3

👉 ❌ Should NOT open

# 🔹 PART 4 – MODIFY ACL 120 (ICMP)

📍 Path:

R1 → CLI

## 💻 👉 ADD

configure terminal

access-list 120 permit icmp any any echo-reply

access-list 120 permit icmp any any unreachable

end

## 🔍 VERIFY

📍 PC-A:

ping 192.168.2.1

👉 ✔️ Should work now

# 🔹 PART 5 – ACL 110 (R3)

📍 Path:

R3 → CLI

## 💻 👉 TYPE

enable

configure terminal

access-list 110 permit 192.168.3.0 0.0.0.255

access-list 110 deny any

interface g0/1

ip access-group 110 in

end

# 🔹 PART 6 – ACL 100 (R3)

📍 Path:

R3 → CLI

## 💻 👉 TYPE

enable

configure terminal

access-list 100 deny ip 10.0.0.0 0.255.255.255 any

access-list 100 deny ip 172.16.0.0 0.15.255.255 any

access-list 100 deny ip 192.168.0.0 0.0.255.255 any

access-list 100 deny ip 127.0.0.0 0.255.255.255 any

access-list 100 deny ip 224.0.0.0 15.255.255.255 any

access-list 100 permit tcp 10.0.0.0 0.255.255.255 host 192.168.3.3 eq 22

interface s0/0/1

ip access-group 100 in

end

# 🔍 FINAL VERIFY

## ✔️ Ping Test (Should FAIL)

📍 PC-C:

ping 192.168.1.3

👉 ❌ Block (as per ACL)

## ✔️ SSH Test (Should WORK)

ssh -l SSHadmin 192.168.2.1

👉 ✔️ Allowed

# 🎯 FINAL RESULT CHECK

| Test | Result |
| --- | --- |
| PC-C → SSH | ✔️ Allowed |
| PC-A → SSH | ❌ Block |
| HTTPS to PC-A | ❌ Block |
| Ping rules | ✔️ Controlled |

# 🔥 FINAL UNDERSTANDING

👉 ACL = Network Security Filter
👉 Har router par lagakar:

- Unauthorized access ❌ block

- Required traffic ✔️ allow

If you want, I can give you
✔️ short exam answer (10 marks ready) 👍