# 🔥 Packet Tracer – ZPF Firewall (ONLY WHAT YOU NEED)

# 🔹 PART 1 – R3 CONFIGURATION

📍 Path:
👉 Click R3 → CLI

## ✅ Step 1: Enable Security License

👉 Type:

enable

configure terminal

license boot module c1900 technology-package securityk9

end

write memory

reload

👉 After reload again go to:

📍 R3 → CLI

## 🔹 Step 2: Create Zones

📍 Path: R3 → CLI

enable

configure terminal

zone security IN-ZONE

exit

zone security OUT-ZONE

exit

## 🔹 Step 3: Create ACL

📍 Path: R3 → CLI

access-list 101 permit ip 192.168.3.0 0.0.0.255 any

## 🔹 Step 4: Create Class Map

📍 Path: R3 → CLI

class-map type inspect match-all IN-NET-CLASS-MAP

match access-group 101

exit

## 🔹 Step 5: Create Policy Map

📍 Path: R3 → CLI

policy-map type inspect IN-2-OUT-PMAP

class type inspect IN-NET-CLASS-MAP

inspect

exit

exit

## 🔹 Step 6: Create Zone Pair

📍 Path: R3 → CLI

zone-pair security IN-2-OUT-ZPAIR source IN-ZONE destination OUT-ZONE

service-policy type inspect IN-2-OUT-PMAP

exit

## 🔹 Step 7: Assign Interfaces to Zones

📍 Path: R3 → CLI

interface g0/1

zone-member security IN-ZONE

exit

interface s0/0/1

zone-member security OUT-ZONE

exit

## 🔹 Step 8: Save Config

📍 Path: R3 → CLI

end

write memory

# 🔥 TESTING (VERY IMPORTANT)

## ✅ TEST 1 (INTERNAL → EXTERNAL)

📍 Path: PC-C → Desktop → Command Prompt

👉 Type:

ping 192.168.1.3

✔️ Should work ✅

👉 SSH Test:

ssh -l Admin 10.2.2.2

✔️ Password: Adminpa55
✔️ Should work ✅

👉 Web Test:

📍 PC-C → Desktop → Web Browser

👉 Enter:

192.168.1.3

✔️ Should open ✅

# ❌ TEST 2 (EXTERNAL → INTERNAL)

📍 Path: PC-A → Desktop → Command Prompt

👉 Type:

ping 192.168.3.3

❌ Should FAIL (Request timeout) ✅

📍 Path: R2 → CLI

👉 Type:

ping 192.168.3.3

❌ Should FAIL ✅

# 🔥 SIMPLE EXPLANATION (END)

- IN-ZONE = internal network (PC-C side)

- OUT-ZONE = external network (PC-A side)

- We allow inside → outside traffic

- We block outside → inside traffic

👉 So:

- PC-C can access everything ✅

- PC-A cannot access PC-C ❌

If you want, I can give you diagram + trick to remember ZPF in 1 minute 👍