# 🔹 PART 1 – ROOT BRIDGE CONFIG

## 📍 Path:

Central Switch → CLI

👉 Type:

enable

configure terminal

spanning-tree vlan 1 root primary

end

## 📍 Path:

SW-1 → CLI

👉 Type:

enable

configure terminal

spanning-tree vlan 1 root secondary

end

## 📍 VERIFY

show spanning-tree

# 🔹 PART 2 – STP SECURITY

## 🔹 STEP 1 – PORTFAST

📍 Path:

SW-A → CLI

👉 Type:

enable

configure terminal

interface range f0/1 - 22

spanning-tree portfast

end

📍 Path:

SW-B → CLI

👉 Type:

enable

configure terminal

interface range f0/1 - 22

spanning-tree portfast

end

## 🔹 STEP 2 – BPDU GUARD

📍 Path:

SW-A → CLI

configure terminal

interface range f0/1 - 22

spanning-tree bpduguard enable

end

📍 Path:

SW-B → CLI

configure terminal

interface range f0/1 - 22

spanning-tree bpduguard enable

end

## 🔹 STEP 3 – ROOT GUARD

📍 Path:

SW-1 → CLI

enable

configure terminal

interface f0/23

spanning-tree guard root

interface f0/24

spanning-tree guard root

end

📍 Path:

SW-2 → CLI

enable

configure terminal

interface f0/23

spanning-tree guard root

interface f0/24

spanning-tree guard root

end

# 🔹 PART 3 – PORT SECURITY

## 🔹 STEP 1 – ENABLE PORT SECURITY

📍 Path:

SW-A → CLI

enable

configure terminal

interface range f0/1 - 22

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security violation shutdown

end

📍 Path:

SW-B → CLI

enable

configure terminal

interface range f0/1 - 22

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security violation shutdown

end

## 🔹 VERIFY PORT SECURITY

📍 Path:

SW-A → CLI

show port-security interface f0/1

# 🔹 STEP 3 – DISABLE UNUSED PORTS

📍 Path:

SW-A / SW-B → CLI

configure terminal

interface range f0/23 - 24

shutdown

end

# 🔍 FINAL VERIFICATION

## ✔️ STP CHECK

show spanning-tree

## ✔️ PORT SECURITY CHECK

show port-security interface f0/1

## ✔️ TEST

📍 PC → Command Prompt

ping (other PC IP)

# 🧠 FINAL EXPLANATION (Simple Hienglish)

👉 Root Bridge

- Central ko root banaya → network stable

👉 PortFast

- PC ports fast start hote hai

👉 BPDU Guard

- Fake switch connect hone se bachata hai

👉 Root Guard

- Koi dusra switch root na ban jaye

👉 Port Security

- Ek port par limited MAC (2)

- Extra device connect → port shutdown

👉 Disable Ports

- Unused ports band = security increase

# 🎯 FINAL RESULT

✔️ Network secure
✔️ STP attack prevent
✔️ CAM overflow prevent
✔️ 100% score possible

If you want, I can give
🔥 common viva questions + answers for this lab 👍