# 🔥 PART 1 – BASIC TEST (MUST DO FIRST)

## ✅ TEST 1

📍 Path: PC-A → Desktop → Command Prompt

ping 192.168.3.3

✔️ Should work

## ✅ TEST 2 (SSH)

📍 Path: PC-C → Desktop → Command Prompt

ssh -l Admin 10.2.2.2

👉 Password: Adminpa55
✔️ Should login → then type:

exit

## ✅ TEST 3 (WEB)

📍 Path: PC-C → Desktop → Web Browser

👉 Enter:

192.168.1.3

✔️ Page should open

# 🔥 PART 2 – R3 CONFIGURATION (MAIN PART)

## 📍 Path: Click R3 → CLI

## ✅ STEP 1 – Enable Security License

enable

configure terminal

license boot module c1900 technology-package securityk9

end

write memory

reload

👉 After reload → again go to R3 CLI

## ✅ STEP 2 – Create Zones

📍 R3 → CLI

enable

configure terminal

zone security IN-ZONE

exit

zone security OUT-ZONE

exit

## ✅ STEP 3 – Create ACL

📍 R3 → CLI

access-list 101 permit ip 192.168.3.0 0.0.0.255 any

## ✅ STEP 4 – Class Map

📍 R3 → CLI

class-map type inspect match-all IN-NET-CLASS-MAP

match access-group 101

exit

## ✅ STEP 5 – Policy Map

📍 R3 → CLI

policy-map type inspect IN-2-OUT-PMAP

class type inspect IN-NET-CLASS-MAP

inspect

exit

exit

## ✅ STEP 6 – Zone Pair

📍 R3 → CLI

zone-pair security IN-2-OUT-ZPAIR source IN-ZONE destination OUT-ZONE

service-policy type inspect IN-2-OUT-PMAP

exit

## ✅ STEP 7 – Assign Interfaces (VERY IMPORTANT)

📍 R3 → CLI

interface g0/1

zone-member security IN-ZONE

exit

interface s0/0/1

zone-member security OUT-ZONE

exit

## ✅ STEP 8 – SAVE

📍 R3 → CLI

end

write memory

# 🔥 PART 3 – FINAL TEST (VERY IMPORTANT FOR 100%)

## ✅ TEST 1 (INSIDE → OUTSIDE)

📍 PC-C → Command Prompt

ping 192.168.1.3

✔️ SUCCESS

ssh -l Admin 10.2.2.2

✔️ SUCCESS

📍 PC-C → Web Browser

👉 Enter:

192.168.1.3

✔️ SUCCESS

## ✅ TEST 2 (OUTSIDE → INSIDE)

📍 PC-A → Command Prompt

ping 192.168.3.3

❌ MUST FAIL (Request timeout)

📍 R2 → CLI

ping 192.168.3.3

❌ MUST FAIL

# 🔥 BONUS (CHECK FOR MARKS)

📍 R3 → CLI

show policy-map type inspect zone-pair sessions

✔️ You will see active sessions (for marks)

# 🔥 FINAL SIMPLE EXPLANATION

- IN-ZONE = PC-C side (internal)

- OUT-ZONE = PC-A side (external)

- Policy = allow inside traffic, block outside traffic

👉 Result:

- PC-C → allowed ✅

- PC-A → blocked ❌

If still not getting 100%, tell me your error screenshot — I will fix it step-by-step 👍